| DMM ENGINEERING | DESIGN -INŽENJERING- WORKS Serbia, 18000 Nis, Street Bulevar Nemanjica no. 27/11 phone/ fax: 018/530-750; 533- 295; e-mail: dmm_nis@yahoo.com |
| --- | --- |
| IDP | CONCEPTUAL DESIGN FOR YARD DECORATION AND IMPROVEMENT OF PRACTICAL TEACHING IN TECHNICAL SCHOOLS “12. FEBRUAR”, “15. MAJ”, “NEIMAR” AND “COLLEGE OD APPLIED TECHNICAL SIENCES” IN NIS |

| Name of the Purchaser on behalf of Republic of Serbia: | - “College of Applied Technical Sciences” in Nis, Street Aleksandra Medvedeva No. 20, Nis - Technical school “15. maj”, Street Beogradska no. 22 - Technical school “12. februar”, Street Beogradska no. 22 |  |  |
| --- | --- | --- | --- |
| Purpose of object and location: | Improvement of practical teaching. space in the courtyard of technical schools Street Aleksandra Medvedeva No. 20, Nis |  |  |
| Class and Category: | Clasification number -211201 | Category- G |  |
| Parts of technical documentation: | 0- Main book 2/2 Planning yard project (IDP) |  |  |
| Project ID / Date: | 142/12 |  | 05/12/15 |
| Responsible person/ Chief designer: | dr Dragan Peric |  | dr Dragan Peric License number: 315 0982 03 |
| Number of copies: |  |  |  |

THE CONTENT OF CONCEPTUAL DESIGN

CONCEPTUAL DESIGN FOR YARD DECORATION AND IMPROVEMENT OF PRACTICAL TEACHING IN TECHNICAL SCHOOLS “12. FEBRUAR”, “15. MAJ”, “NEIMAR” AND “COLLEGE OD APPLIED TECHNICAL SIENCES” IN NIS

0. MAIN BOOK						 ACCORDING TO ANNEX 1

0.1. Front page of the Main book

0.2 The content of the Main book

0.5 The content of technical documentation

0.6 Information on designers

0.7 General information about the project

0.8 Enclosures (which are not required by regulations)

* Project task

2/2 PLANNING YARD PROJECT 				ACCORDING TO ANNEX 9

2/2.1. Front page of the Yard project

2/2.2. The content of the Project

2/2.3 Decision determining the Chief designer

2/2.4 Chief designer statement

2/2.5 Textual documentation

2/2.6 Numeric documentation

2/2.7 Graphic documentation

Bill of works

for works on the yard and improvement of  practical teaching in technical schools

01. Prior works

01.001	"Renewing and marking of the yard before and during constructions"

m1	4.492,24х		=

01.002	Machine stripping of existing concrete slabs d=10cmm, with loading and transportation up to 5km.

m2	2.464,16х		=

01.003	Machine demolition of existing cocnrete curbs of different dimensions, with loading and transportation up to 5km.

m1	280,61	х		=

01.004	Machine demolition of existing cocnrete paths of different dimensions, with loading and transportation up to 5km.

m2	856,63	х		=

Total prior works:

2. Ground works

02.001	Machine excavation of humus with loading and transportation up to 5km to landfill

m3	272,58	х		=

02.002	Machine excavation of III and IV category materials with loading and transportation up to 5km to landfill

m3	1.759,49 х		=

02.003	Machine excavation of III and IV category materials for  atmospheric sewerage trench with loading and transportation up to 5km to landfill

m3	155,00   х		=

02.004	Machine excavation of III and IV category materials for drains and drains connection with atmospheric sewagewith with  loading and transportation up to 5km to landfill

m3	235,94	х		=

02.005	Machine excavation of III and IV category materials for drains and drains connection with atmospheric sewagewith with  loading and transportation up to 5km to landfill

m3	24,00	х		=

02.006	Subgrade rolling

m2	4.362,18х	               =

Total ground works:

03. Concrete works

03.001	"Leveling of existing manhole to needed height. Manhole lid previously concrete into reinforced concrete slab MB30 dimensions  120*120*15cm. Billing by piece."

piece	2,00	х		=

03.002	Purchase, transportation and installation of grey cocnrete curbs 12/18cm, on the concrete layer MB 15

m1	399,65	х		=

03.003	Purchase, transportation and installation of grey cocnrete drain flumes 40х40х6 cm according to detail

m1	102,32	х		=

03.004	Purchase, transportation and installation of concrete MB-30 for production of  base on planned contents for practical teaching according to details from the project.

m1	24,60	х		=

03.005	Production of inspection manhole from reinforced concrete rings Ø  1000/500mm

piece	4,00	х		=

03.006	Purchase and installation of finishing conical elemenet of manhole  Ø 1000/600mm

piece	2,00	х		=

03.007	Production of non reinforced oversite concrete of inspection manhole, circular 150/25cm, with half-round upper surface that will be the bottom of the inspection manhole.

m3	0,75	x		=

03.008	Purchase, transportation and installation of two times ribbed polyethylene sewer pipes  Ø 300mm

m1	155,00	х		=

03.009	Purchase, transportation and montage of cast iron manhole covers with circular hole frames Ø 600mm with ventilation holes for  heavy traffic load

piece	2,00	х		=

03.010	Purchase and montage of iron manhole climbers  Ø 25mm on every 50 cm with protection					piece	8,00	х		=

03.011	Purchase, transportation and montage of concrete pipes  Ø 450mm for drains on the concrete layer MB20  d= 10cm

m1	20,00	х		=

03.012	Testing of pipeline sewerage network for water tightness before their backfilling. 50% of total length of the pipeline is being tested.

m1	77,50	х		=

03.013	Purchase, transportation and installation of iron catchment basin dimensions 46/63 х 8 cm					piece	10,00	х		=

03.014	Purchase, transportation and installation of PVC pipes  Ø150 mm between drains and manhole 					m1	179,95	х		=

Total concrete works:

04. Superstructure

04.001	Production of crushed stone material binding layer 0-31,5mm, d=10cmn, with purchase and transportation up to 35km and compression to wanted compactness

m3	360,86	х		=

04.002	Production of pebble layer  d=15 cm,  purchase and transportation up to 35km and compression to wanted compactness

m3	557,59	х		=

04.003	Production of stamped concrete installation of fine graded concrete C 25/30 (MB 30) reinforced with polypropylene fibers, d = 10cm  and steel fabric Q-196 with cutting dilatation joints width  0,5cm and height 4-5cm on field maximally 25 m2 with backfilling with badieon in color and varnishing with colorless epoxide lacquer, in all according to project and technical conditions

m2	3681,91x		=

Total superstructure:

05.1. Plateau with stands

05.1.1.	Purchase, transportation and installation of concrete MB30 reinforced with reinforced mesh  Ø6 in needed  sheeting, Inner radius of the plateau is  4.5m, and outer 7м, in all according to sketch. Note:  Напомена: substrate is processed trough bill of quantities for whole yard.

m3	12,00	х		=

05.1.2.	Processing of all visible surfaces with stamped concrete, in  the pattern of choice of the investor					m2	10,00	х		=

Total plateau with stands:

05.2. Solar watch

05.2.1.	"Purchase and production of mold for dial for solar watch, for production over the stamped concrete.

Make the mold according to drawing from the project."

piece	1,00	х		=

05.2.2.	Production of the dial into the stamped concrete d=10cm, according to drawing

m2	1,00	х		=

05.2.3.	Purchase, transportation and installation of stainless steel holders and clockhands Ø50, processed by grinding to the mirror effect, in all according to cketch. Place clockhands into foundation pits  30х0х40  concrete MB30

piece	1,00	х		=

Total solar watch:

05.3. House of shadows

05.3.1.	"Purchase, transportation and installation of cocnrete  MB 30 into oversite concrete for mechanism for turning on pebble layer 10cm, reinforced wirh reinforcement mesh  Р-335"						m3	1,00	х		=

05.3.2.	Purchase and installation of mechanism for turning of the plateau by investors orders.

piece	1,00	х		=

05.3.3.	Purchase, transportation and installation of carriers for plateau made out of thick wall pipes square section 10х10. Make carrier according to drawing. Leave a connection in a form of a box height 10cm for placing a house of shadows pillar and for anchoring. Paing the carriers with base and finish color. Around the edges bind carriers with sheet metal thick 3mm and high 15mm processed the same as the carriers.

кg	300,00	х		=

05.3.4.	Purchase, transportation and installation of fir boards into the floor of platau thick 5cm, tailor according to cheme for hexagon according to deawing, proces the board to be smooth, coat with protective coating and finally overpaint with lacquier for boats in 3 layers.

m3	2,00	х		=

05.3.5.	"Purchase, transportation and installation of bearing construction of the object, poles 14х14 horizontal carriers dimensions 18х14, in all according to drawing, processed and grated to needed smoothness, protected with priotective coating and finishing coating with lacquiet for boats in 3 layers."					m3	1,00	х		=

05.3.6.	Purchase, transportation and installation of sheet metal d=3mm  with cut openers according to drawing, painted with protective and finishing paint by the investors choice, attaching for bearing construction and execute staffs with L profiles  5х5.

m2	10,00	х		=

05.3.7.	Purchase, transportation and installation of wooden staffs dimensions 5х8, processed and protected as in previous positions.

m1	100,00	х		=

Total house of shadows:

05.4. Аerodynamic elevator

05.4.1.	Manual excavation of grounds for poles of the elevator

m3	2,00	х		=

05.4.2.	Purchase, transportation and installation of  concrete MB25, into foundation pits, on all four sides of the pit and in the bottom place ribbed reinforcement mesh R-335, and place anchorages  m20 -4pieces, for attaching of poles and base plates. At the bottom of the pit place pebble d=10cm.

m3	2,00	х		=

05.4.3.	Purchase, transportation and installation of  poles for elevator  Ø159,d=4.5, lenght 6м, at the bottom weld vertical nodal sheet metal high 250м d=5mm, for connection of the pole and base plate  400х400, d=10mm, attach to already installed anchorages and strengthen with coresponding screws and coasters.  Paint the construction with base and finishing coating in two layers. Production of pole and base pole and montage is based on the sketch of the designer. Billing by kg of installed steel.

кg	400,00	х		=

05.4.4.	Purchase, transportation and installation of steel plasticized cables Ø10 according to sketch. Binding the cables to poles and mutually do according to technical regulations.

m1	100,00	х		=

05.4.5.	Purchase and installation of plastic  propeller dimensions  50 cm and placing the cable before bracing

piece	54,00	х		=

Total aerodynamic elevator:

05.5. Cylindrical mirror with a curved bench

05.5.1.	Purchase of materials, production and montage of stainless steel cylinder, treatment - sanded to mirror effect diameter 1м and heihgt 2м (above base), wall thick 4mm.

piece	1,00	х		=

05.5.2.	Purchase of materials, production and montage of curved prohrom benches, Набавка материјала, израда  и уградња закривљених клупа од прохрома, curvature radius 2.4m

piece	2,00	х		=

05.5.3.	Purchase, trasportation and installation of concrete MB15 for watering and fixating of benches and mirrors.

m3	0,50	х		=

05.5.4.	Purchase, trasport and installation of stamped concrete d=10cm on the plateau by the mirrors					m2	18,00	х		=

Total cylindrical mirror with a curved bench:

05.6. Sound shell 1 and 2

05.6.1.	Purchase, production and montage of concrete sound shells, made in moldm, according to designers' sketch, reinforced with ribbed armature Ø10. Final treatment with white cement, cement mortar 1:2

m3	0,50	х		=

05.6.2.	Purchase, production and montage of concrete sound shells, made in moldm, according to designers' sketch, reinforced with ribbed armature Ø10. Final treatment with white cement, cement mortar 1:2

piece	2,00	х		=

05.6.3.	Purchase, trasportation and installation of stamped concrete on the plateau around the shells in design chosen by the investor.

m2	15,00	х		=

Total sound shell 1 and 2:

05.7. Marquee with solar panels

05.7.1.	Purchase, trasportation and installation of concrete  MB30 into footings, for wooden construction of the marquee.

m3	1,50	х		=

05.7.2.	Purchase, production and installation of metal construction of the marquee with carrier for placement of solar pannels. Make the construction out of steel profiles: poles- boxes 10х10 cm, box frame 50х30 cm, pannels carriers L profile 30х30 mm, steel sheet d= 1 mm. Make the steel construction according to details in project, paint with protective color and finish color in two layers. Weld the steel construction according to regulations for this kind of works.

kg	400,00	х		=

05.7.3.	Purchase and installation of solar panels according to the manufacturer's instructions.  Panels power 250W

piece	8,00	х		=

05.7.4.	Purchase of the convertors for turning solar to electric energy and installation according to the manufacturer's instructions and technical regulations for this kind of works.

piece	1,00	х		=

05.7.5.	Purchase and placement of spotlights which are powered by the solar energy to the network builted by seel cable  Ø6mm, that binds to existing poles. Do the binding of the spotlights according to technical regulations.

piece	15,00	х		=

05.7.6.	Purchase of steel cables Ø6mm and placing them above the yard. Bind the cables to existing installation poles.

m1	400,00	х		=

Total marquee with solar panels:

05.8. Outdoor fitness

05.8.1.	Machine excavation of third category grounds d=30cm, with loading and transport up to 5km					m3	25,00	х		=

05.8.2.	Manual excavation of third category grounds for footings of the fitness equipment, with transportation to the landfill up to 5m, with loading and transport up to 5km

m3	2,00	х		  =

05.8.3.	Production of pebble layer  d=15cm in a compacted state, purchase, transport and installation					m3	12,00	х		=

05.8.4.	Purchase, transport and installation of crushed stone aggregate layer 0-31mm thick 10cm in a compacted state

m3	7,50	х		=

05.8.5.	Purchase, transport and installation of concrete  MB25 into footings for fitness equipment, and for info board on before tamponed pebble base  d=10cm

m3	2,00	х		=

05.8.6.	Purchase, transport and installation of  curbes  12/18 on concrete MB15 layer

m1	40,00	х		=

05.8.7.	Purchase, transport and installation of fitness equipment						0,00	0,00

twister			piece	1,00	х		=

rowing simulator	piece	1,00	х		=

riding simulator		piece	1,00	х		=

skiing simulator		piece	1,00	х		=

walking simulator	piece	1,00	х		=

shaft			piece	1,00	х		=

device for abdominal muscles	piece	1,00	х		=

05.8.8.	Purchase, transport and installation of concrete  MB25d=10cm on the plateau for fitness equipment.

m2	75,00	х		=

05.8.9.	Purchase, transport and installation of stamped concrete below every fitness equipmnet and below info board.

m2	26,00	х		=

05.8.10.Purchase, transport and installation of info board, make the frame out of steel profiles 50х50, height 2м, wide 1,2м; made the info board out of multilayered plywood dimensions 1,2х1,0  and place on the profile frame, and user manual for using the fitness equippment place on multilayered. Paint the frame with protective and finish color.

piece	1,00	х		=

Total outdoor fitness:

05.9. Drinking fountain 1 and 2

05.9.1.	Manual excavation of grounds for foundations of the drinking fountain and manhole (ViK)					m3	2,00	х		=

05.9.2.	Concrete of manhole for ViK mesh thick 10cm, in all according to drawing, reinforce with reinforcement mesh Q84. Purchase, transportation and installation of concrete is covered with this position.

m3	1,50	х		=

05.9.3.	 Purchase, transportation and installation of concrete into foundations of the drinking fountains and  trough. Make the trough according to deawing and reinforce it with Q84.

m3	1,00	х		=

05.9.4.	Purchase, transportation and installation of pipes for water supply and sewerage with all necessary fittings, purchase of faucets all according to sketch

piece	2,00	х		=

05.9.5.	Generator of color due to water flow

piece	2,00	х		=

Total drinking fountain 1 and 2

05.10. Benches for sitting and waste baskets

05.10.1.	Producion and placement of concrete siting bench with wooden seat and support according to drawing from the project. Seat and suppport are  made out of boards 8х5cm, with finishing treatment

piece	5,00	х		=

05.10.2.	Producion and placement of concrete siting bench with wooden seat without support according to drawing from the project. Seat is made out of boards 8х5cm, with finishing treatment, protection and varnishing.

piece	10,00	х		=

05.10.3.	Purchase and placing waste baskets in all according to drawing

piece	10,00	х		=

Total benches for sitting and waste baskets:

05.11. Sports grounds equipment

05.11.1.	Purchase of goals with net for small football playground

piece	2,00	х		=

05.11.2.	Purchase and installation of carriers for baskketball baskets in pacage with table and basket hoop with net.

piece	2,00	х		=

05.11.3.	Labelling football and basketball playground in all according to applicable regulations.						piece	1,00	х		=

05.11.4.	Purchase and delivery of mobile fence out of metal montage pannels with footings for fixating and apropriate binders

m1	100,00	х		=

05.11.5.	Purchase and installation of spotlights around the playground on to existing street lighting

piece	4,00	х		=

Total sports grounds equipment:

05.12. Other works

05.12.1.	Production of thematic molds for stamped concrete for marked signage of every school and printing them on the terrain

piece	4,00	х		=

05.12.2.	Purchase and installation of video surveillance system in schoolyard - 8 cameras with all following equipment and specialist equipment for picture archiving.

piece	1,00	х		=

05.12.3.	Greening of the free surface of the yard

m2	800,00	х		=

05.12.4.	Purchase and placing equpment for  Wi-Fi , Skype and Viber

piece	1,00	х		=

Total other works:

R E C A P I T U L A T I O N

01. Prior works

02. Ground works

03. Concrete works

04. Superstructure

05. 05. Auxiliary facilities for practical teaching

05.1. Plateau with stands

05.2. Solar watch

05.3. House of shadows

05.4. Аerodynamic elevator

05.5. Cylindrical mirror with a curved bench

05.6. Sound shell 1 and 2

05.7. Marquee with solar panels

05.8. Outdoor fitness

05.9. Drinking fountain 1 and 2

05.10. Benches for sitting and waste baskets

05.11. Sports grounds equipment

05.12. Other works

TOTAL

VAT 20%

M A D E    B Y

dr Dragan Peric, construction engeneer